## Generic Risk Assessment for use of: Mobile Elevated Work Platforms (‘MEWP’s) (FMRA. 0003) 									July 2017

For a MEWP to be used safely and in accordance with Estates Facilities’ policy the ‘Controls’ below must be applied by employees, and complete a specific risk assessment of the work to be undertaken prior to use of the MEWP:

Estates Facilities employees must not use MEWP unless they are holders of a current, valid International Powered Access Federation (IPAF) PAL (Powered Access Licence) card, suitably experienced and fit to undertake the tasks required of them.  Supervisors should look beyond MEWP training and not overlook the requirement for operators to have task-specific training and equipment (e.g. use of a chainsaw and all that entails). Staff must comply with the Estates Facilities’ permit-to-work system.

Managers/Supervisors must ensure there is a suitable level of supervision to ensure safe systems of work are specified, appropriate and being used throughout the work procedure.  Establish agreed systems of communication (e.g. between MEWP operators and banksman, emergency services).    Estates Facilities’ Managers/Supervisors should be particularly vigilant in respect of alternative MEWPs supplied in replacement for the ones specified; ensure we have a system for recording and responding promptly to reported faults, repairs and maintenance requirements.

Hiring MEWPs – Where it is deemed necessary to hire a MEWP it will only be operated by the Hire Company.  A risk assessment must be produced to cover its mobilisation, operation, inspection and any storage.

| ## Hazard | Persons at Risk | Existing Controls | Action Needed |
| --- | --- | --- | --- |
| Mobilising MEWP to work location | Operator/Workforce/Pedestrians/building fabric | This will only be carried out under a Permit to Work | Consideration must be given to the route / method which will be used to move a MEWP to and from its designated work location. The route and method must be approved by the Maintenance Team Manager who will provide the Permit to Work. |
| Equipment maintenance | College / Operators | Any MEWP owned and operated by the College will appear on the asset register and be regularly inspected and maintained in accordance with the Lifting Equipment and Lifting Operations Regulations (LOLER) 1998 | Prior to the operation / use of a MEWP it must be visually checked by a holder of the IPAF licence who will also check that it has been fully inspected under LOLER. |
| ## Hazard | Persons at Risk | Existing Controls | Action Needed |
| Potential for MEWP to overturn. Uneven, slippery, sloping or soft ground, excavations | Operators/Workforce | Do not use on uneven, slippery, sloping or soft ground. The ground may have to be compacted and levelled in advance; check that all the tyres are properly inflated. | Any outriggers are to be extended and chocked as necessary before raising the platform. Check the work area for localised features, e.g. manholes, temporary covers, service ducts, potholes, etc. (a hole 75 mm deep can cause an overturn). Establish the load bearing capacity (general and point loading, e.g. outriggers) when working inside a building or on a structure. |
| Equipment failure | Operators | Estates Facilities staff must be trained to know what to do if the machine fails with the platform in the raised position. There must be an agreed and understood rescue plan that will safely recover workers from a raised platform. |  |
| Moving MEWP whilst platform raised | Operators/Workforce | The MEWP is not to be moved with the platform in the raised position unless the equipment is designed to allow this to be done safely (check the manufacturer's instructions). |  |
| Falls from platforms | Operators | Operators to wear a fall restraint lanyard and harness attached to the platform | Allowing people to climb out of the carrier (i.e. the basket or cage) whilst in the raised position is not recommended. |
| Over-reaching | Operators/Workforce | Operators must maintain both feet on the working platform. Steps or any other form of podium are not to be used. |  |

| Hazard | Persons at Risk | Existing Controls | Action Needed |
| --- | --- | --- | --- |
| Overloading MEWP carrier | Operator/Workforce | Do not exceed the specified loading rating of the MEWP. Think of combined operators’ weights, plus their tools plus the weight of materials used. | Is the maximum loading weight displayed in the carrier? |
| Contact with overhead obstacles | Operator/Workforce | The identification of overhead obstacles will be included in the risk assessment undertaken prior to its use.. | Do operators know how to position the MEWP for optimum use? Ensure operators know when further operation would be unsafe. |
| Falling objects | Pedestrians / Workforce below | The MEWP is provided with guard rails and toe boards or other suitable barriers. Cordon off working area if necessary. | Use tools attached by lanyard. |
| Personal injury | Operator/Workforce | Wear appropriate Personal Protective Equipment (PPE) and sturdy footwear. | None. |
| Using MEWP as a crane | Operator/Workforce/Pedestrians | Using MEWP as a crane is forbidden. | None. |
| Adverse Weather | Operator/Workforce | Do not work at height in storms or strong winds. Ensure operators know when further operation would be unsafe. | None. |

I have read and understood the above risk assessment and received appropriate relevant training:

Employee’s Signature: ___________________    Employee’s Name (print): ____________________ Date: ___________________

Risk Assessment Signed Off by:   Steve Hughes    Date: July 2017 Next review date: July 2018