## Generic Risk Assessment for use of: Stepladders (FMRA. 0002)

For a stepladder to be used safely and in accordance with Estates Facilities policy the ‘Controls’ below  must be applied by employees:

Estates Facilities employees must not use stepladders unless they have attended the College’s Ladder Safety Training Course.

Prior to using a stepladder a visual inspection must be carried out to include checking for: damage to the stiles and rungs, presence of safety feet and locks, damage to the platform and no obvious loose rivets or fixings. The stepladder must have an in-date ScaffTag attached.   Employees must position the stepladder at right angles to the work being undertaken.  You must not work sideways on a stepladder as any force you apply to the work item may be transferred through your body and cause the ladder to topple.

Stepladders must not be used for work of a duration greater than 15 to 30 minutes and may only be used for low risk tasks.

Employees must read HSE’s Safe Use of Ladders and Stepladders: http://www.hse.gov.uk/pubns/indg455.htm

| ## Hazard | Persons at Risk | Existing Controls | Action Needed |
| --- | --- | --- | --- |
| Equipment quality | Operator | Check condition of steps prior to working – Check treads, feet of steps, rivets, screws, platform, locks, handholds, etc. No Class 3 (domestic) stepladders are to be used. | Only industrial grade stepladders which have been inspected and included on the College’s ladder register are to be used. Step Ladders must be Class 1 (the highest rated in terms of strength and quality). These ladders are suitable for use in heavy duty industrial applications and environments. BS EN 131 ladders (previously ‘Class 2’) may be used for light work. |
| Uneven, sloping or soft ground | Operators/Workforce | Do not use steps on uneven, sloping or soft ground. | Consider other methods of access e.g. hydraulic lift platform, ladder or podium steps |
| Height | Operators | Do not use steps for works higher than that recommended for the steps supplied. If going above 4 steps high, steps to be footed by second person. | Operators are advised not to use the top two rungs of the steps and are not to use the top platform for work unless it is designed with special handholds, and avoid over-reaching as the stepladder is liable to overturn if you do. |
| Slippery ground | Operators | Do not use steps if possible on slippery ground or floors – dry them where possible. | None. |
| ## Hazard | Persons at Risk | Existing Controls | Action Needed |
| Adverse Weather | Operator/Workforce | Do not work at height in storms or strong winds | Cease work if weather deteriorates. |
| Overloading ladder | Operator/Workforce | Only one person on the stepladder at any one time, do not exceed loading rating of steps. |  |
| Lighting | Operators | Do not carry out operations in poorly lit areas. | Provide additional lighting – extension leads, torches etc. |
| Falling objects | Operator/ Pedestrians | Be aware of falling/dropped objects from above. Cordon off working area if necessary. | None. |
| Protection/Slipping | Operator/Workforce | Wear appropriate Personal Protective Equipment (PPE) and sturdy footwear. | None. |
| Over-reaching | Operators/Workforce | Do not over-reach. Work only within safe reaching distance and keep your belt buckle within the styles of the stepladder. . | None. |
| Obstacles – overhead or in close proximity to the ladder | Operator/Workforce | Correct stepladder positioning. | If possible remove obstacle prior to equipment use. Consider other methods of access e.g. hydraulic lift platform, ladder. |
| Carrying items (e.g. goods/tools) up step ladder | Operator/Workforce, persons passing | Steps to be footed by second work person. Use lifting appliances such as hoist lines or ropes. | Use a tool belt. Be wary of awkward/heavy loads, changes to centre of gravity as load is raised. |
| Electrical Hazards | Operator/Workforce | Avoid Electrical Hazards. | Turn off power. |
| Ladder disturbed | Operator/Workforce | Avoid placing stepladders where they may be struck by doors opening, persons passing, vehicles etc. | Step ladders can tip over very easily if struck on the side. |
| Cramped conditions | Operator/Workforce | Correct stepladder positioning, use alternative equipment. | Always open step ladder fully and lock into position. |

I have read and understood the above risk assessment and received appropriate relevant training:

Employee’s Signature: _____________________     Employee’s Name (print): ____________________ Date: ___________________

Risk Assessment Signed Off by:   Steve Hughes    Date: July 2017 Next review date: July 2018