RISK ASSESSMENT AND STANDARD OPERATING PROCEDURE

| 1. PERSON CARRYING OUT ASSESSMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Nanet Willumsen |  | Position |  |  | Research Technician |  |  | Date |  | 22/06/2020 |  |  |
| 2. DESCRIPTION OF ACTIVITY (include storage, transport and disposal if relevant) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RNAscope on formalin fixed paraffin embedded tissue (https://acdbio.com/technical-support/user-manuals) Note: Nitrile dissolves after prolonged exposure to xylene. Either wear xylene resistant gloves or use tweezers while working with xylene to reduce exposure time. DAY 1 - Bake FFPE slides in dry oven for 1hr at 60oc. Optional: slides can be left at RT O/N. - Prepare oven and reagents: Set HybEZ oven to 40 oc Warm humidity tray containing wet humidifying paper for 30mins before use. Prepare 200ml fresh 1X target retrieval and heat in steamer in a plastic pot with foil lid - Use within 30mins of boiling. - In fumehood deparraffinize sections: 100% xylene 2x 5min, 100% ethanol 2 x 1 min - Air dry slides 5min at RT - Apply 5-8 drops Hydrogen peroxide to each tissue section for 10mins at RT. - Place slides into slide rack submerged in distilled H20, agitate and repeat wash. Target retrieval - Dip slides in warmed H20 - Using forceps, submerge slide rack in into boiling 1X target retrieval solution for ‘tissue appropriate time’ (15-30 minutes). - Immediately transfer into distilled H20, agitate and repeat wash. - Wash in 100% ethanol for 3mins - Air dry slides (can do at 60 oc) - Draw hydrophobic barrier on slides. Optional: slides can left at RT O/N. - Place slides in HybEZ slide rack and add ~5 drops Protease Plus. - Place rack in humidity chamber at 40 oc for 30mins. - During incubation, warm 50X wash buffer at 40 oc for 10-20mins. - Prepare 3L of 1X wash buffer: 2.94L distilled H20 and 1 bottle (60mls) of 50X wash buffer (1:50 final dilution), mix well. Prepare probes - If probes not prepared then: Warm probes for 10min at 40 oc, then cool to RT. Briefly spin the C2 probe. Mix 1:50 ratio of the C2 probe to C1 probe by pipetting 1 volume of C2 probe to 50 volumes of C1 probe in a tube. Invert several times. Store 4 oc up to 6months Hybridize probes - Remove excess liquid from slides, place in HybEZ slide rack and add ~4 drops of probe to each section. - Insert the HybEZ slide rack into the oven for 2hrs at 40 oc. - Wash slides in 1X wash buffer for 2min at RT, repeat. - Dilute20X SSC buffer to 5X in H20, add to slides and leave slides in SSC buffer O/N. DAY 2 Hybridize Amps - Remove excess liquid from slides. Add ~4 drops Amp 1 to each section - Place in oven for 30mins at 40 oc - Wash in 1X wash buffer for 2min at RT, repeat wash. - Remove excess liquid from slides. Add ~4 drops Amp 2 to each section - Place in oven for 15mins at 40 oc - Wash in 1X wash buffer for 2min at RT, repeat wash. - Remove excess liquid from slides. Add ~4 drops Amp 3 to each section - Place in oven for 30mins at 40 oc - Wash in 1X wash buffer for 2min at RT, repeat wash. - Remove excess liquid from slides. Add ~4 drops Amp 4 to each section - Place in oven for 15mins at 40 oc - Wash in 1X wash buffer for 2min at RT, repeat wash. - Remove excess liquid from slides. Add ~4 drops Amp 5 to each section - Place in oven for 30mins at RT. - Wash in 1X wash buffer for 2min at RT, repeat wash. - Remove excess liquid from slides. Add ~4 drops Amp 6 to each section - Place in oven for 15mins at RT. - Wash in 1X wash buffer for 2min at RT, repeat wash. Detect Signal - Briefly spin RED-B and mix 1 volume of RED-B to 60 volumes of RED-A (must use within 3-5mins). For example: add 2.5ul of RED-B to 150ul RED-A per section. - Removes excess liquid from slides, add ~120ul RED solution to each section. - Incubate in sealed HybEZ slide rack for 10min at RT. - Wash in 1X wash buffer for 2min at RT, repeat wash. Continue hybridization - Remove excess liquid, add ~4 drops Amp 7 to each section. - Incubate for 15mins at 40 oc - Wash in 1X wash buffer for 2min at RT, repeat wash. - Remove excess liquid, add ~4 drops Amp 8 to each section. - Incubate for 30mins at 40 oc - Wash in 1X wash buffer for 2min at RT, repeat wash. - Remove excess liquid, add ~4 drops Amp 9 to each section. - Incubate for 30mins at RT - Wash in 1X wash buffer for 2min at RT, repeat wash. - Remove excess liquid, add ~4 drops Amp 10 to each section. - Incubate for 15mins at RT - Wash in 1X wash buffer for 2min at RT, repeat wash. Detect signal - Briefly spin GREEN-B and mix 1 volume of GREEN-B to 50 volumes of GREEN-A (must use within 3-5mins). For example: add 3ul of GREEN-B to 150ul of GREEN-A per section. - Remove excess liquid from slides, add ~120ul of GREEN solution to each section. - Incubate in sealed HybEZ slide rack for 10mins at RT. - Remove solution from slides and wash in 1X wash buffer for 5mins. - Wash quickly with distilled H20. Counterstain - Place slides in 50% Gills Hematoxylin for 30 sec at RT. (1:1 dilution in H20). - Wash 3-5 times in tap water. - In fume hood: dilute 28% ammonia to 0.02% ammonia in H20 (143ul + 185.7mls H20) Wash slides for 10 sec in 0.02% ammonia water then wash 3-5 times in tap water. - Dry slides at 60 oc for 15mins - In fume hood: Dip the slides in fresh 100% xylene and immediately place 1-2 drops EcoMount on the slide before the xylene dries, coverslip the section. - Leave to dry for 30min in fume hood. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. LOCATION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Campus | Hammersmith |  | Building |  |  | Burlington Danes |  |  | Room |  | BD floor 4 |  |  |
| 4. HAZARD SUMMARY |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Accessibility |  | X |  |  |  |  | Mechanical |  |  |  | X |  |  |
| Manual Handling |  | X |  |  |  |  | Hazardous Substances |  |  |  | Y |  |  |
| Electrical |  | X |  |  |  |  | Noise |  |  |  | X |  |  |
| Working at height |  | X |  |  |  |  | Extreme temperature |  |  |  | Y |  |  |
| Falling objects |  | X |  |  |  |  | Pressure/steam |  |  |  | Y |  |  |
| Trip hazards |  | X |  |  |  |  | Other |  |  |  | X |  |  |
| Lone Working Permitted? |  | Yes No |  |  |  |  | Permit-to-Work required for planned maintenance? |  |  |  | Yes No N/A |  |  |
| 5. Who might be harmed and how? |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Staff / students |  |  |  |  |  |  | Cleaners, engineers etc |  |  |  |  |  |  |
| Support staff |  |  |  |  |  |  | Other |  |  |  |  |  |  |
| 6. How often is the process being carried out? |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Once a day Once a week Once a month Every 6 months Annually Other – give details |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7. Brief description of the procedure |  |  |  |  |  |  | Precautions (Controls) in place |  |  |  |  |  | Is risk high, medium or low? |
| Xylene |  |  |  |  |  |  | Wear PPE and use in a chemical fume hood. Nitrile gloves will dissolve in xylene. Dispose via chemical waste route only. Store in fume cabinet separate to flammable solvents, keep away from sources of ignition. |  |  |  |  |  | L |
| Ethanol |  |  |  |  |  |  | Wear PPE. Store locked in flammable cupboard. Avoid use near flames, heat or sparks. |  |  |  |  |  | L |
| RNAscope Protease Plus |  |  |  |  |  |  | Wear PPE. Provided and used in small volumes reducing any risk. |  |  |  |  |  | L |
| Ammonium hydroxide (28%) |  |  |  |  |  |  | Wear PPE. Use in Fume hood. Do not dispose of 28% ammonium hydroxide down the drain. Diluted to working solution of 0.02% posing minimal risk. Dispose 0.02% down the drain with excess water. |  |  |  |  |  | L |
| EcoMount (EM897) |  |  |  |  |  |  | Wear PPE. EcoMount will be used in small volumes in a fume hood when mounting slides greatly reducing any risk. |  |  |  |  |  | L |
| Amp and detection probes |  |  |  |  |  |  | Wear PPE. Probes contain low % of hazardous chemicals minimising risk. The probes are provided in small volumes and used in a very small volume therefore pose no great risk. DO NOT USE IF PREGNANT. AMP1 and AMP 3 contain Formamide and have the H360D code – May damage the unborn child.– contact Occupational health for advice. |  |  |  |  |  | L |
| Steamer |  |  |  |  |  |  | Wear PPE, keep away from hot steam outlets. Use forceps to transfer racks into boiling solutions. Do not use glass in steamer. Allow solutions that are too hot to cool before handling/disposing. Do not place steamer directly under plug socket. |  |  |  |  |  | L |
| 8. Are extra precautions needed? If no please tick box and move onto next section |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If yes, please describe |  |  |  | Who has been asked to do this? |  |  |  |  |  | By what date? |  |  |  |
| 9. EMERGENCY ACTIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| First Aid treatment for skin contact with xylene, ethanol, RNAscope protease plus, ammonium hyrodxide (28%) and EcoMount: Remove any contaminated clothing. Rinse skin well with water for a prolonged period. If necessary seek medical attention. First Aid treatment for inhalation: Remove person to a well ventilated area. If person seems to become dizzy or loses consciousness call security and wait for emergency services to arrive. First Aid treatment for eye splash: Rinse thoroughly for at least 15min using the eye wash station. If necessary, seek medical attention. First Aid treatment for ingestion: Do NOT induce vomiting. Rinse mouth with water and seek medical attention immediately. First Aid for scalds (from steam): Remove from heat source. Cool burn with cool water for >20mins, removing any clothing near burn unless stuck to skin. Can cover with cling film. Seek medical attention if burn is major. Chemical Spillage The spill kits are in E432 (near to the fume cupboards and in E533 - wash room) Within the laboratory but outside any primary containment facility such as a fume cupboard: If the spillage is small <2ml, clean up with paper towel and water. See Local Rules. If spillage is Xylene and is large, evacuate immediate area. Notify laboratory manager and building manager. Spill will need to be cleaned up using the chemical spill kit. Facemask, lab coat and xylene resistant gloves must be worn. Dispose of waste in yellow chemical waste bag provided and send through Imperial hazardous waste route. If spillage is ethanol and is large, use chemical spill kit immediately. Within a fume cupboard (if relevant): If there’s a spillage in the fume cupboard. Lower sash as low as possible. Clean spill using chemical spill kit. Dispose of via Imperial hazardous waste route. De-contaminate area with water and paper towel. Emergency support through security: 4444 (+442075891000) In all instances of accident OR near miss, notify the safety department and complete a SALUS report. https://www.imperial.ac.uk/safety/safety-by-topic/accidents--incidents/ Occupational health contacts: https://www.imperial.ac.uk/occupational-health/ email: occhealth@imperial.ac.uk phone: +44 20 7594 9401 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. Monitor and review |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Controls should be monitored: daily weekly monthly 6 monthly annually other I will review this risk assessment at least every 6 months every 12 months Immediately in the event of process / location change or incident or accident |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11. Training record – use this section to record the names and date of any persons you are training in this risk assessment and associated procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  | Date |  |  | Name |  |  |  | Date |  |

https://www.imperial.ac.uk/safety/forms/ for all specific risk assessment forms.